SITE ACCESS SURVEY

Company Name:

Company Contact Name

Address:

Post Code

Tel:                                                                        Type of Business

Date:                                                                      Number of Employees

KEY:

Y = YES

N = NO

M = MEASURE & RECORD HERE

IF YOU ARE IN DOUBT PLACE A TICK IN THE N BOX

| ###### Area of Consideration |  |  |  |
| --- | --- | --- | --- |
| 1.Approach to Site | Y | M | N |
| 1. Is the approach to the entrance in its own private grounds? If not tick the N box and proceed to the next section |  |  |  |
| 2. Is the approach within 100m walking distance of public transport? |  |  |  |
| 3. Does the route go across any roads or near the top of external stairs? |  |  |  |
| 4. Is the route clearly marked indicating the route for disabled people to the disabled entrance? |  |  |  |
| 5. Is the route identified by visual, audible and tactile information? |  |  |  |
| 6. Is the route on an incline? |  |  |  |
| 7. Is the route free of kerbs? |  |  |  |
| 8. Is the route surface smooth and slip resistant? |  |  |  |
| 9. Is the route wide enough? (measure average width of walkway) |  |  |  |
| 10. Is the route free from hazards such as building features, litterbins, overhanging branches, and sandwich and advertising boards? |  |  |  |
| 11. Is the route lighting free of shadows? |  |  |  |
| 12. Is seating provided along the route if it is in excess of 50m? |  |  |  |
| 2.Access to Site | Y | M | N |
| 1. Do the gates / barriers (if any) to the site open automatically? |  |  |  |
| 2. Is there a passenger dropping off and collection area for cars? |  |  |  |
| 3. Is there a passenger dropping off and collection area for minibuses or coaches? |  |  |  |
| 3.Parking | Y | M | N |
| 1. Does the site have a car park? If not tick the N box and proceed to the next section |  |  |  |
| 2. Are there at least 3 (or 6% of total capacity whichever is greater) designated car parking spaces for the disabled? |  |  |  |
| 3. Are the designated spaces slightly larger than standard spaces? (measure width and length)) |  |  |  |
| 4. Are the designated spaces more than 100m from the facilities they serve? |  |  |  |
| 5. Are the designated spaces clearly signposted from the car park entrance? |  |  |  |
| 6. Are the spaces clearly marked as provision for disabled drivers or passengers only? |  |  |  |
| 7. Is the car park surface smooth, slip resistant, firm, level, well drained and free from loose stones? |  |  |  |
| 8. Are the routes from the designated spaces to the building accessible and unobstructed? |  |  |  |
| 9. Is the car park lighting free of shadows? |  |  |  |
| 10. Are car park meters, and ticket bay machines accessible to wheelchair users? (measure the height of coin slots and ticket drawers) |  |  |  |
| 4.Route from Car Park/Public Transport Stations | Y | M | N |
| 1. Is the route clearly marked indicating the route for disabled people to the main entrance and the disabled entrance? |  |  |  |
| 2. Is the route identifiable by visual, audible and tactile information? |  |  |  |
| 3. Is the route on an incline? |  |  |  |
| 4. Is the route to the building free from kerbs? |  |  |  |
| 5. Is the surface smooth, slip resistant, firm, level, well drained and free from loose stones? |  |  |  |
| 6. Is the route wide enough? (measure the average width of the route) |  |  |  |
| 7. Is the route free from hazards such as building features, litter bins, street furniture and overhanging branches, and advertising boards |  |  |  |
| 8. Do windows open outwards into an access route at a height of less than 1100mm? |  |  |  |
| 9. Is the route lighting free of shadows? |  |  |  |
| 10. Is seating provided along the route if the route is over 50m or on an incline? |  |  |  |
| 5.External Ramps | Y | M | N |
| 1. Is there a ramp? If not tick the N box and proceed to the next section |  |  |  |
| 2. Is the ramp fixed? |  |  |  |
| 3. Is the ramp portable? |  |  |  |
| 4. Is the portable ramp stored nearby and is it easy to access? |  |  |  |
| 5. Is the ramp accompanied by steps for ambulant disabled persons? |  |  |  |
| 6. Does the ramp surface contrast with the paving and landings? |  |  |  |
| 7. Is the ramp gradient suitable for all users? (measure the height of the steps and also record the number of steps) |  |  |  |
| 8. Are the landings of adequate size for wheelchair users to circulate? |  |  |  |
| 9. Are adequate handrails provided to both sides? (measure the width of the handrail and the gap between the handrail and the wall) |  |  |  |
| 10. Are the handrails / grab rails fixed to the wall? (measure the height of the rails off the floor) |  |  |  |
| 11. Are there side rails to the edge of the ramp for the safety of wheelchairs? |  |  |  |
| 12. Is the ramp lighting free of shadows? |  |  |  |
| External Steps | Y | M | N |
| 1. Are there external steps? If not tick the N box and proceed to next section |  |  |  |
| 2. Are the step treads all the same length? (measure) |  |  |  |
| 3. Are the steps all the same height? (measure) |  |  |  |
| 4. Are the nosings, which are 2-inch strips painted or attached to the front and top of each step, readily identifiable and unlikely to create trip hazards? |  |  |  |
| 5. Are adequate handrails provided to both sides of the ramp? Required if 2 or more risers (measure the width of the handrail and the gap between the handrail and the wall) |  |  |  |
| 6. Are visual and tactile warning signs provided at the top and bottom of the steps on the ground and on the walls? |  |  |  |
| 7. Are the steps lit and free of shadows? |  |  |  |
| 6.Main Entrance | Y | M | N |
| 1. Is the main entrance easy to find? |  |  |  |
| 2. Is the entrance distinguishable from the building front? |  |  |  |
| 3. Is the entrance well lit and free from shadows? |  |  |  |
| 4. Is a porch or covered area provided at the entrance door? |  |  |  |
| 5. Do the entry phones and / or security system allow use by people with sensory and / or mobility impairments? Are they easy to use? (measure height off floor) |  |  |  |
| 7.Main Entrance Door | Y | M | N |
| 1. Is the opening of the main entrance door an adequate width? (measure) |  |  |  |
| 2. Is there clear signage indicting that there is an alternative disabled entrance? (measure height of signage off floor) |  |  |  |
| 3. If fully glazed is the glass on the main entrance door clearly marked especially for the visually impaired? |  |  |  |
| 4. Can people either side of the door, either seated or standing see each other and be seen? |  |  |  |
| 5. Is there space alongside the leading edge of the door for a wheelchair user to pass through easily before negotiating further doors? (measure the space between the leading edge and the nearest fixed obstacle ) |  |  |  |
| 6. Is the door handle at an accessible height and is it easy to grip and easy to operate? (measure height off floor) |  |  |  |
| 7. Is the door closer BS compliant? |  |  |  |
| 8. Can the door be opened with ease? |  |  |  |
| 9. Is there a telecom or security entry system in place? |  |  |  |
| 10. Is the threshold flush or level with the surface flooring? (measure the height of the threshold ) |  |  |  |
| 11. Is the threshold weather mat of firm texture? |  |  |  |
| 12. Is the threshold weather mat flush with the floor? |  |  |  |
| 13. Is the main entrance door a fire exit door? |  |  |  |
| 7a. Alternative Disabled Entrance |  |  |  |
| 1. Is there an alternative disabled entrance? |  |  |  |
| 2. If fully glazed is the glass on the disabled entrance door clearly marked especially for the visually impaired? |  |  |  |
| 3. Can people either side of the door, either seated or standing see each other and be seen? |  |  |  |
| 4. Is there space alongside the leading edge of the door for a wheelchair user to pass through easily before negotiating further doors? (measure the space between the leading edge and the nearest fixed obstacle ) |  |  |  |
| 5. Is the threshold flush or level with the surface flooring? (measure the height of the threshold ) |  |  |  |
| 6. Is the door handle at an accessible height and is it easy to grip and easy to operate? (measure height off floor) |  |  |  |
| 7. Is the door closer BS compliant? |  |  |  |
| 8. Can the door be opened with ease? |  |  |  |
| 9. Is the alternative door always unlocked during business hours? |  |  |  |
| 10. Is the threshold weather mat of firm texture? |  |  |  |
| 11. Is the threshold weather mat flush with the floor? |  |  |  |
| 12.Is there a telecom or security entry system in place? |  |  |  |
| 8.Automatic Door | Y | M | N |
| 1. Is there an automatic door? If not tick the N box and proceed to next section |  |  |  |
| 2. Is it a sliding automatic door? |  |  |  |
| 3. Does the door remain open long enough for a slow moving person to pass through? Time the opening and closing speeds |  |  |  |
| 4. Does the door have safety sensors? |  |  |  |
| 5. Does the door have both visual and tactile warning information? |  |  |  |
| 6. Is there a revolving door supplemented by a normal door? |  |  |  |
| 9.Lobby | Y | M | N |
| 1. Is there a lobby? If no tick the N box and proceed to next section |  |  |  |
| 2. Does the inner door meet with the same criteria as the entrance door in width? (measure width of the door space) |  |  |  |
| 3. Is the lobby large enough for a wheelchair user to clear one door before negotiating the second? (measure depth) |  |  |  |
| 4. Is there a clear view into the lobby from outside? |  |  |  |
| 5. Are signs designated and positioned to inform the visually impaired and wheelchair users with reduced eye level? |  |  |  |
| 6. Does the transitional lighting ensure that people can adjust to different lighting levels? |  |  |  |
| 7. Are the floor surfaces slip resistant? |  |  |  |
| 8. Do hard floor surfaces cause excessive echo? |  |  |  |
| 9. Are floor surfaces firm for wheelchair maneuver? |  |  |  |
| 10. Are junctions between floor surfaces formed to prevent tripping hazards or visual confusion? |  |  |  |
| 10.Reception | Y | M | N |
| 1. Is there a reception area? If no tick the N box and proceed to next section |  |  |  |
| 2. Is the reception desk easily identified from the entrance doors? |  |  |  |
| 3. Is the route from the main entrance to the reception desk free of obstacles? |  |  |  |
| 4. Is the reception desk / checkout area suitable for approach and use from both a standing and sitting position? (measure height of counter) |  |  |  |
| 5. Can customers fill out forms, cheques etc.with ease? Is there room to achieve a comfortable, steady writing position |  |  |  |
| 6. Is the desk glare free? |  |  |  |
| 7. Are signs designated and located for visually impaired and wheelchair using customers with reduced eye level? (measure height off floor) |  |  |  |
| 8. Is there an induction loop fitted or other hearing enhancement system? |  |  |  |
| 9. Is there a sliding glass window on the reception desk? |  |  |  |
| 10. Does the background noise stop 2 people talking and hearing each other comfortably when stood 2m apart? |  |  |  |
| 11. Is the lighting free of shadows? |  |  |  |
| 12. Do floor and wall finishes contrast in colour? |  |  |  |
| 13. Is a telephone provided? |  |  |  |
| 14. Is the telephone at a height suitable for all users? (measure height off floor) |  |  |  |
| 15. Are the telephones fitted with inductive couplings for earring aid users? |  |  |  |
| 16. Are all the floor surfaces slip resistant? |  |  |  |
| 17. Do hard surface floors cause excessive echo? |  |  |  |
| 18. Are floor surfaces firm for wheelchair maneuver? |  |  |  |
| 19. Are junctions between floor surfaces formed to prevent tripping hazards or visual confusion? |  |  |  |
| 20. Is there adequate space for wheelchair users to wait? (measure) |  |  |  |
| 21. Is there adequate space for wheelchair users to sit alongside a seated companion? (measure) |  |  |  |
| 11.Corridors | Y | M | N |
| 1. Are there corridors? If no tick the N box and proceed to next section |  |  |  |
| 2. Are corridors, passageways and aisles wide enough for wheelchair users and others to pass through? (measure the width of the corridor) |  |  |  |
| 3. Are the corridors free from hazards, fixed or portable, which are obstacles to wheelchair users and people with impaired vision? |  |  |  |
| 4. If obstructions cannot be removed e.g. radiators, are they guarded? |  |  |  |
| 5. Is there adequate turning space for wheelchair users? (measure) |  |  |  |
| 6. Does both the natural and artificial light avoid reflection, glare, shadows and silhouette? |  |  |  |
| 7. Are wall finishes roof finishes and floor finishes all in contrasting colours to avoid visual confusion? |  |  |  |
| 8. Are there adequate visual signs to avoid disorientation? |  |  |  |
| 9. Are direction or information signs (including means of escape) clearly visible from both a standing and seated position? |  |  |  |
| 10. Is seating provided along the route? |  |  |  |
| 11. Do hard floor surfaces cause excessive echo? |  |  |  |
| 12. Are floor surfaces firm for wheelchair maneuver? |  |  |  |
| 13. Are junctions between floor surfaces formed to avoid tripping hazards and visual confusion due to bold colours? |  |  |  |
| 12.Internal Lobbies | Y | M | N |
| 1. Are there any internal lobbies? If no tick the N box and proceed to next section |  |  |  |
| 2. Is there a clear view into the lobby from the outside? |  |  |  |
| 3. Is the lighting free of shadows? |  |  |  |
| 4. Do inner door widths meet the required criteria? (measure width) |  |  |  |
| 5. Is there adequate space for wheelchair uses to clear the first door before negotiating the second? (measure the clearance of the door leading edge to the next door) |  |  |  |
| 6. Are floor surfaces slip resistant? |  |  |  |
| 7. Are floor surfaces firm for wheelchair maneuver? |  |  |  |
| 8. Are junctions between floor surfaces formed to prevent tripping hazards and visual confusion? |  |  |  |
| 13.Assembly / Waiting Areas | Y | M | N |
| 1. Is there an assembly area? If no tick the N box and proceed to next section |  |  |  |
| 2. Is the lighting free of shadows? |  |  |  |
| 3. Is there sufficient circulation space for wheelchairs? (measure) |  |  |  |
| 4. Is the area free from obstructions which are either fixed or portable? |  |  |  |
| 5. Are there seating arrangements in a variety of styles, which are suitable for people with disabilities? |  |  |  |
| 6. Are all areas for assembly fitted with an induction loop? |  |  |  |
| 7. If use of an induction loop is precluded, is there an infrared system? |  |  |  |
| 8. Are there telephones fitted with induction loop couplers? |  |  |  |
| 9. Are text telephones available? For deaf and hard of hearing people? |  |  |  |
| 10. Are controls, switches, handles, fixtures and fittings within easy reach for all people including wheelchair users? (measure height off floor) |  |  |  |
| 14.Internal Ramps | Y | M | N |
| 1. Is there an internal ramp? If no tick the N box and proceed to next section |  |  |  |
| 2. Is the ramp portable? |  |  |  |
| 3. Is the ramp accompanied by steps for ambulant disabled persons? |  |  |  |
| 4. Is the ramp wide enough for wheelchair users and the gradient suitable? (measure height of steps and record the number of steps) |  |  |  |
| 5. Are adequate handrails provided to both sides of the ramp? Required if 2 or more risers (measure the width of the handrail and the gap between the handrail and the wall) |  |  |  |
| 6. Is the ramp surface slip resistant? |  |  |  |
| 15.Internal Steps | Y | M | N |
| 1. Are there internal steps? If no tick the N box and proceed to next section |  |  |  |
| 2. Are the treads all the same length? (measure) |  |  |  |
| 3. Are the risers all the same height? (measure) |  |  |  |
| 4. Are the nosings, which are 2-inch strips painted or attached to the front and top of each step, readily identifiable and unlikely to create trip hazards? |  |  |  |
| 5. Are adequate handrails provided to both sides? Required if 3 or more risers |  |  |  |
| 6. Are visual and tactile warning signs situated at the top and bottom of the steps on the ground and on the walls? (measure height of visual signs off floor) |  |  |  |
| 16.Internal Stairs | Y | M | N |
| 1. Are there internal staircases? If no tick the N box and proceed to next section |  |  |  |
| 2. Is the location of the stairs clearly indicated? |  |  |  |
| 3. Are the treads all the same length? (measure) |  |  |  |
| 4. Are the risers all the same height? (measure) |  |  |  |
| 5. Are the nosings, which are 2-inch strips painted or attached to the front and top of each step, readily identifiable and unlikely to create trip hazards? |  |  |  |
| 6. Are adequate handrails provided to both sides? (measure the width of the handrail and the gap between the handrail and the wall) |  |  |  |
| 7. Are visual and tactile warning signs situated at the top and bottom of the staircase? (measure height of visual signs off floor) |  |  |  |
| 8. Are landings long enough and provided at intermediate levels on long flights? (measure) |  |  |  |
| 9. Is the lighting on the stairs free of shadows? |  |  |  |
| 17.Elevators | Y | M | N |
| 1. Are products or services offered on more than one storey in the building? |  |  |  |
| 2. Is there a lift? If no tick the N box and proceed to next section |  |  |  |
| 3. Can the lift be used as an “Evacuation Lift”? |  |  |  |
| 4. Is the location of all lifts clearly defined? |  |  |  |
| 5. Is the opening of the lift door suitable for wheelchair users? (measure width of door opening) |  |  |  |
| 6 Are the car dimensions sufficient to allow use and access by all users? (measure width and depth) |  |  |  |
| 7. Are there support rails in the car appropriately designed and situated? (measure height off floor) |  |  |  |
| 8. Is there a delayed action closing system which can be overridden? |  |  |  |
| 9. Are the controls inside the car (including the Emergency call system) within easy reach of all users? (measure height off floor) |  |  |  |
| 10. Are all signs designated using good colour and/or tonal contrast between the lettering and its background? |  |  |  |
| 11. Is lighting in the lift free of shadows? |  |  |  |
| 12. Is there voice indication of floor reached? |  |  |  |
| 13. Are there call controls and floor indicators at each landing? |  |  |  |
| 14. Is there adequate and unobstructed entry at each landing and lift entry for wheelchair users? |  |  |  |
| 18.Platform Lifts | Y | M | N |
| 1. Is there a platform lift? If no tick the N box and proceed to next section |  |  |  |
| 2. Can the platform/stair lift be safely approached by wheelchair users at both the top and bottom? (free of obstacles) |  |  |  |
| 3. Is the platform of adequate size for wheelchair users? (measure width and breadth) |  |  |  |
| 4. Are controls clearly identifiable and within convenient reach of standing and seated users? |  |  |  |
| 5. In the event of power failure or emergency does the platform automatically return to lower level and allow egress? |  |  |  |
| 19.Stair Lifts | Y |  | N |
| 1. Is there a stair lift? If no tick the N box and proceed to next section |  |  |  |
| 2. Can the stair lift be safely approached by wheelchair users at both top and bottom? (it needs to be free of obstacles) |  |  |  |
| 3. Is the platform of adequate size for wheelchair user transfer? |  |  |  |
| 4. Is the approach safe at all landings? |  |  |  |
| 5. In the event of power failure or emergency does the platform automatically return to the lower level and allow egress? |  |  |  |
| 6. When not in use does the platform automatically revert to folded position to ensure there is no obstruction on the stairs? |  |  |  |
| 7. Does the stair lift have a "soft start" action? |  |  |  |
| 20.Internal Doors | Y | M | N |
| 1. Are all doors absolutely necessary for safety or functional reasons? |  |  |  |
| 2. Are the internal doors easily distinguishable from their surroundings? |  |  |  |
| 3. Are glass doors clearly visible when closed? |  |  |  |
| 4. Can people on both sides of the door see each other and be clearly seen? |  |  |  |
| 5. Is the opening of the door a sufficient width for wheelchair users? (measure width of opening) |  |  |  |
| 6. Is there adequate space alongside the leading edge of the door for a wheelchair user to pass through? (measure space) |  |  |  |
| 7. Is the door control at a height suitable for both standing and seated users? (measure height off floor) |  |  |  |
| 8. Are the controls clearly distinguishable from the door itself? |  |  |  |
| 9. Are the controls easily gripped and operated? (measure height off floor) |  |  |  |
| 10. Are suitable entry phones and security systems provided? |  |  |  |
| 11. Is there a door closer attached? |  |  |  |
| 21.Ambulant WC Provision | Y | M | N |
| 1. Is there WC provision for ambulant people with disabilities? If no tick the N box and proceed to next section |  |  |  |
| 2. Is there adequate space alongside the leading edge of the door for a wheelchair user to pass through? (measure) |  |  |  |
| 3. Is the WC door easily negotiated by wheelchair users from either the left or right side? |  |  |  |
| 4. Is the approach to the cubicle free of steps, narrow corridors and obstructions? |  |  |  |
| 5. Is there sufficient space for wheelchair users to maneuver and open doors inside the WC and pass through? (measure leading edge space) |  |  |  |
| 6. Are all fittings readily distinguishable from their background? |  |  |  |
| 7. Are the compartment door controls easily gripped and operated? (measure height off floor) |  |  |  |
| 8. Can ambulant disabled people maneuver and rise and lower themselves in a standard cubicle? |  |  |  |
| 9. Do the floors have a slip resistant finish? |  |  |  |
| 10. Is the travel distance to a WC no greater than for able bodied users? |  |  |  |
| 22.WC Provision for Disabled Users | Y | M | N |
| 1. Is provision made for wheelchair users and is it clearly signed? If no tick the N box and proceed to next section |  |  |  |
| 2. Is the approach free of obstructions? |  |  |  |
| 3. Is the location of the disabled toilets clearly signed using good colour and/or tonal contrast between the lettering and its background, and between the signboard and its background. |  |  |  |
| 4. Is the compartment large enough to allow maneuvering of wheelchairs into position for frontal, lateral, angled and backward transfer both unassisted and assisted? (measure dimensions width and breadth) |  |  |  |
| 5. Are door fittings and locks, easily reached and operated? (measure height off floor) |  |  |  |
| 6. Is there an emergency call system with someone designated to respond and can it be operated from floor level? |  |  |  |
| 7. Are hand washing and drying facilities within easy reach of someone seated on the WC? |  |  |  |
| 8. Are all basin fittings suitable for people who cannot grip? |  |  |  |
| 9. Are all fittings readily distinguishable from their background? |  |  |  |
| 10. If there is more than one standard layout WC compartment provided, are they able to offer a left-sided approach and a right-sided approach? |  |  |  |
| 11. Is there adequate space either side of the WC pan for assisted use for wheelchair users? (measure) |  |  |  |
| 12. Does the floor have a slip resistant finish? |  |  |  |
| 13. Is the toilet seat height at an ideal height for transfer from wheelchairs? (measure height) |  |  |  |
| 14. Are fire alarms present in the area? |  |  |  |
| 15. Are there plastic coated grab rails and hand rails? |  |  |  |
| 23.Changing/ Shower Facilities | Y | M | N |
| 1. Are there changing facilities? If no tick the N box and proceed to next section |  |  |  |
| 2. Is a shower compartment provided which is suitable for wheelchair users?(level access) |  |  |  |
| 3. Is the height of the showerhead variable? |  |  |  |
| 4. Have a tip-up seat and suitable support rails been provided? |  |  |  |
| 5. Is there a dressing cubicle and does it comply with required size and layout? (measure) |  |  |  |
| 6. Are lockers easily reached and operated by all users including wheelchair users? |  |  |  |
| 7. Does the floor have a slip resistant finish? |  |  |  |
| 8. Are all fittings readily distinguishable from their background? |  |  |  |
| 24.Bathrooms | Y |  | N |
| 1. Is there a bathroom? If no tick the N box and proceed to next section |  |  |  |
| 2. Is the bathroom provided with fittings suitable for use by people with disabilities? The surface finish of all fittings and grab bars should contrast visually with the wall and floor finishes and should be easy to grip |  |  |  |
| 3. Is the bathroom ensuite? |  |  |  |
| 4. Does the bathroom comply with the required size and layout? (measure) |  |  |  |
| 5. Are all fittings easily reached? (measure height off floor) |  |  |  |
| 6. Are all fittings readily distinguishable from their background? |  |  |  |
| 7. Does the floor have a slip resistant finish? |  |  |  |
| 8. Is a hoist provided for use with the bath? |  |  |  |
| 25.Counters/ Service Desk | Y | M | N |
| 1. Are there counters or service desks? If no tick the N box and proceed to next section |  |  |  |
| 2. Is there provision and room on both sides of the counter or desks for wheelchair users? |  |  |  |
| 3. Are induction loops fitted at counters with glazed screens or where there is background noise? |  |  |  |
| 4. Are counters designed or positioned to avoid glare or silhouetting? |  |  |  |
| 26.Front of Counter / General Serving / Retail Area | Y | M | N |
| 1. Is the route from the main entrance to the counter free of obstacles? |  |  |  |
| 2. Is the counter assessable to users with sufficient space to allow documents to be signed? |  |  |  |
| 3. Is assistance for the mobility-impaired offered? |  |  |  |
| 4. Is assistance for the sight-impaired offered? |  |  |  |
| 5. Is assistance for the hearing-impaired offered? |  |  |  |
| 6. Is assistance for the dexterity-impaired offered? |  |  |  |
| 7. Is assistance for the mentally impaired offered? |  |  |  |
| 8. Is assistance for wheelchair-users offered? |  |  |  |
| 9. Is staff awareness training provided regarding offering assistance? |  |  |  |
| 10. Is disability etiquette training provided to staff? |  |  |  |
| 11. Is seating provided? |  |  |  |
| 12. Are the seats provided at intervals along internal routes or where waiting is likely? |  |  |  |
| 13. Are the seats stable, with and without arm rests, and are they provided in a range of heights? |  |  |  |
| 14. Is there sufficient space for wheelchair users to sit alongside a seated companion? (measure) |  |  |  |
| 15. Do tables allow for wheelchair access? (measure height and knee space) |  |  |  |
| 16. Are the display stands fixed to the floor or walls for security? |  |  |  |
| 17. Are all goods on display accessible to all? (measure the height above floor level of the highest item) |  |  |  |
| 18. Is the lighting free of shadows? |  |  |  |
| 19. Is there sufficient circulation space for wheelchairs? (measure) |  |  |  |
| 20. Is the area free from obstructions or hazards to wheelchair users and visually impaired persons? |  |  |  |
| 21. Are all areas for assembly fitted with an induction loop? |  |  |  |
| 22. If use of an induction loop is precluded, is there an infra red system? |  |  |  |
| 23. Is the lighting free of shadows? |  |  |  |
| 24. Are the lights positioned where they do not cause glare, reflection, confusing shadows or pools of light & dark? |  |  |  |
| 25. Can the occupiers control the lighting? |  |  |  |
| 26. Is the lighting in the workspace controllable and adjustable to ensure optimum lighting conditions? |  |  |  |
| 27. Is there automatically activated booster lighting for dim conditions? |  |  |  |
| 28. Is fluorescent lighting installed only where it is unlikely to cause inconvenience to people with hearing impairments (background noise and electric signals)? |  |  |  |
| 29. Is the environment free of unnecessary noise? |  |  |  |
| 31. Is there a good balance of hard and soft surfaces? |  |  |  |
| 32. Is table service available? |  |  |  |
| 33. Is there a portable induction loop for waiter service? |  |  |  |
| 34. Are tea, coffee facilities and vending machines accessible and usable by all? (measure height of coin slots etc.) |  |  |  |
| 35. In a Restaurant or Bar what proportion of the area where seating is to be provided is accessible to wheelchair users? (Estimate % area) |  |  |  |
| 36. In a Restaurant or Bar if orders are taken at the bar, what height is the bar? (measure) |  |  |  |
| 37. In a Restaurant or Bar if orders are taken at the bar is an induction loop fitted? |  |  |  |
| 38. Are Braille price lists and menus available? |  |  |  |
| 39. Are large print price lists and menus available? |  |  |  |
| 40. Are all prices clearly displayed at suitable heights? (measure) |  |  |  |
| 41. Are all signs designated using colour and tonal contrast? |  |  |  |
| 42. Do tables allow for wheelchair access? (measure height and knee space) |  |  |  |
| 27.Telephones | Y | M | N |
| 1. Are there public telephones provided? If no tick the N box and proceed to next section |  |  |  |
| 2. Are the telephones fixed at a height that allows easy access by wheelchair users? (measure height off floor to coin slot and receiver) |  |  |  |
| 3. Is a suitable seat available for people who need to sit down whilst using the phone? (measure height off floor) |  |  |  |
| 4. Are the telephones equipped with inductive couplers for hearing aid users? |  |  |  |
| 5. Is a text phone available? |  |  |  |
| 28.Kitchens | Y | M | N |
| 1. Is there a Kitchen? If no tick the N box and proceed to next section At present the act does not apply to commercial kitchens |  |  |  |
| 2. Is the kitchen provided with fittings suitable for use by people with disabilities? (measure height of fittings) |  |  |  |
| 3. Is the kitchen of adequate size and layout for use by people with disabilities? (measure to ensure wheelchair turning space) |  |  |  |
| 4. Are work surfaces adequate for wheelchair users in that there is sufficient knee space? (measure) |  |  |  |
| 5. Can all fixtures, fittings and appliances including switches and controls be easily reached and operated? (measure height) |  |  |  |
| 6. Are all fittings readily distinguishable from their background? |  |  |  |
| 7. Is the kitchen lighting free of shadows and strong lighting? |  |  |  |
| 8. Does the floor have a slip resistant finish? |  |  |  |
| 29.Bedrooms | Y | M | N |
| 1. Are there Bedrooms? If no tick the N box and proceed to next section |  |  |  |
| 2. Which floor are the disability bedrooms on? |  |  |  |
| 3. Are bedrooms provided with fittings that are easy to use and are suitable for use by people with disabilities? |  |  |  |
| 4. Are the bedrooms of adequate size and layout for use by people with disabilities? (measure) |  |  |  |
| 5. Can all fixtures, fittings, switches and controls be easily reached and operated? (measure heights of all fittings) |  |  |  |
| 6. Is lighting in the bedroom free of shadows? |  |  |  |
| 7. Has provision been made for hearing impaired people in case of emergency evacuation? |  |  |  |
| 30.IT accessibility | Y | M | N |
| 1. Do you require staff to use computers? |  |  |  |
| 2. Is the font accessible for visually impaired? |  |  |  |
| 3. Is screen reader software used? |  |  |  |
| 4. Has a risk assessment been carried out on all display screen equipment users? |  |  |  |
| 31.Information and Controls | Y | M | N |
| 1. Is the building equipped to provide hearing assistance? |  |  |  |
| 2. Are signs positioned in logical areas, visibly identifiable and easy to read? |  |  |  |
| 3. Is information also provided in tactile form (such as maps and models, or Braille?) |  |  |  |
| 32.Means of Escape | Y | M | N |
| 1. Is the audible alarm supplemented by a visual system? |  |  |  |
| 2. Are ground floor exit routes as accessible as entrance routes? |  |  |  |
| 3. Is vertical escape from upper or lower floors possible using a fire protected lift with an independent power supply? |  |  |  |
| 4. If people with disabilities cannot completely evacuate the building, can they reach places of safety or refuge? |  |  |  |
| 5. Are all escape routes capable of accommodating wheelchairs? (1200mm) |  |  |  |
| 6. Is there a personal egress plan prepared for any member of staff who may require assistance in an emergency? |  |  |  |
| 7. Are exit routes regularly checked for obstacles? |  |  |  |
| 8. Are alarm systems regularly checked by a qualified engineer? |  |  |  |
| 9. Are fire doors regularly checked by a qualified engineer? |  |  |  |
| 10. Are all fire doors regularly maintained? |  |  |  |
| 11. Is the escape route clearly signed? |  |  |  |
| 12. Are staff members trained in helping mobility-impaired people evacuate? |  |  |  |
| 13. Have ‘buddies’ or ‘helpers’ been nominated to disabled staff members? |  |  |  |
| 14. Are the places of refuge large enough for the projected number of people likely to need them and are evacuation chairs available with trained staff? |  |  |  |
| 15. Has a fire risk assessment been carried out? |  |  |  |
| 33.Building Management | Y | M | N |
| 1. Are external routes (including steps and ramps) kept clear, unobstructed and free from surface water, ice and snow? |  |  |  |
| 2. Are designated car parking spaces only used by disabled drivers and are they kept clear of obstructions? |  |  |  |
| 3. Is all door furniture regularly maintained? |  |  |  |
| 4. Are alternative doors to automatic ones kept unlocked? |  |  |  |
| 5. Are lifts, platforms and stair lifts regularly serviced by qualified engineers? |  |  |  |
| 6. Are disabled WC's kept clear and not used for storage? |  |  |  |
| 7. Are suitable cleaning products used in order that the cleaning and polishing of slip resistant floors does not render them slippery? |  |  |  |
| 8. Are checks made to ensure that junctions between different flooring materials do not become worn presenting a trip hazard? |  |  |  |
| 9. Are considerations made prior to redecoration to maintain a carefully devised colour scheme? |  |  |  |
| 10. Are maps of the building interiors kept up to date? |  |  |  |
| 11. Do new signs integrate effectively with current signage? |  |  |  |
| 12. Are signs replaced after redecoration? |  |  |  |
| 13. Are windows, blinds and lamps kept clean and in working order? |  |  |  |
| 14. Are defective lamps replaced swiftly? |  |  |  |
| 15. Are induction loops and infrared systems advertised and checked regularly? |  |  |  |
| 16. Are air conditioning units maintained to reduce noise? |  |  |  |
| 17. Are exit routes regularly checked for obstacles? |  |  |  |
| 18. Are alarm systems regularly checked by a qualified engineer? |  |  |  |
| 19. Are all new staff trained in emergency response procedures? |  |  |  |
| 20. Is there an escape strategy for visitors who require assistance? |  |  |  |
| 21. Is there a personal egress plan prepared for any member of staff who may require assistance in an emergency? |  |  |  |
| 22. Are both the general escape strategy and personal emergency egress plans effective and efficient? i.e. fire drills |  |  |  |
| 23. Is there an action plan to carry forward information and recommendations from this Access Audit? |  |  |  |

Comments Section

- Have any refurbishments been carried out to the building in the last 10 years? If there has please enter the details below.

PHOTOS REQUIRED

| AREA | PHOTO TAKEN (YES / NO) | COMMENTS |
| --- | --- | --- |
| ENTRY GATES / BARRIERS |  |  |
| CAR PARK FACILITIES |  |  |
| EXTERNAL RAMPS AND / OR STEPS |  |  |
| MAIN ENTRANCE - THRESHOLD STEP AND INTERCOM AND DOOR FURNITURE |  |  |
| ALTERNATIVE ENTRANCE - THRESHOLD STEP AND INTERCOM AND DOOR FURNITURE |  |  |
| ROUTE TO RECEPTION DESK |  |  |
| INTERNAL RAMPS AND STEPS |  |  |
| TOILET FACILITIES |  |  |
| ALL SIGNS (INDUCTION LOOPS ETC) |  |  |
| ALL FIRE EXITS |  |  |
| DESKS |  |  |
| PUBLIC TELEPHONES |  |  |
| SEATING / WAITING AREAS |  |  |
| BEDROOMS |  |  |
| KITCHENS |  |  |
| BATHROOMS |  |  |
| SHEVING – HEIGHT ACCESSIBILITY |  |  |
| AUTOMATIC DOORS |  |  |
| REFRESHMENT MACHINES – SNACKS AND DRINKS – HEIGHT TO COIN SLOT |  |  |