WORK AT HEIGHT RISK ASSESSMENTS

Putting up displays

Accessing filing / storage

Opening / closing windows / adjusting blinds

Changing fluorescent tubes using stepladder

Use of a ladder

Use of a tower scaffold

Work at height within drama

Low roof Access

| ##### GENERIC RISK ASSESSMENT |  |  |  |  |
| --- | --- | --- | --- | --- |
| Establishment: | Assessment by: | Date: |  |  |
| Review Date: | Approved by: | Date: |  |  |
| WORK ACTIVITY (brief description) Putting up displays (low height, approximately 0.5 M above average persons height) using a kick stool |  |  |  |  |

| ## Hazard / Risk | Who is at Risk? | Normal Control Measures (Brief description and/or reference to source of information). | Additional Control Measures (to take account of local/individual circumstances). | Risk Rating H/M/L |
| --- | --- | --- | --- | --- |
| Use of incorrect equipment (desk, chairs etc) Falls, falling objects | Staff Visitors | - Staff instructed not to use makeshift steps (chairs/ tables etc) - Restrict displays to head height where practical - Ensure display boards are accessible - Kick stool provided for users to access display areas safely |  |  |
| Defective equipment / Falls, falling objects | Staff Visitors | - All access equipment suitable for purpose, checked before use and maintained in good condition. - Reporting procedure in place for identified defective items with prompt removal /remedial action. |  |  |
| Inappropriate use of access equipment Overreaching, stretching etc. Uneven floor causing instability Falls, falling objects | Staff Visitors | - Displays prepared as far as possible before putting them up. - Use of access equipment restricted to those trained and competent in use. - Staff to re-position kick stool rather than overstretch. - Floor surfaces suitable to use kick stool ( firm and even surface) - Persons using access equipment to wear appropriate footwear ( low heeled, non slip soles) - Do not leave unattended to avoid unauthorised use. |  |  |

| REVIEWS: |  |  |
| --- | --- | --- |
| DATE OF REVIEW: | REVIEWED BY: | COMMENTS: |
| DATE OF REVIEW: | REVIEWED BY: | COMMENTS: |
| DATE OF REVIEW: | REVIEWED BY: | COMMENTS: |

| ##### GENERIC RISK ASSESSMENT |  |  |  |  |
| --- | --- | --- | --- | --- |
| Establishment: | Assessment by: | Date: |  |  |
| Review Date: | Approved by: | Date: |  |  |
| WORK ACTIVITY (brief description) Accessing office filing and storage on fixed shelving (approximately 1 M above average persons height) using a step ladder |  |  |  |  |

| ## Hazard / Risk | Who is at Risk? | Normal Control Measures (Brief description and/or reference to source of information). | Additional Control Measures (to take account of local/individual circumstances). | Risk Rating H/M/L |
| --- | --- | --- | --- | --- |
| Use of incorrect equipment (desk, chairs etc) Falls, falling objects | Staff Visitors Pupils | - Staff instructed not to use makeshift steps (chairs/ tables etc) - Restrict high level storage - Ensure storage remains accessible - User to ensure access equipment is of adequate height. - Step ladder provided for users to access areas safely (more height is required than can be achieved with a kick stool) |  |  |
| Defective equipment Falls, falling objects | Staff Visitors Pupils | - All access equipment suitable for purpose, checked before use and maintained in good condition. - Frequent documented checks on stepladder to ensure safe working condition ( 6 monthly / termly) - Reporting procedure in place for identified defective items with prompt removal /remedial action. |  |  |
| Inappropriate use of access equipment Overreaching, stretching etc. Uneven floor causing instability Falls, falling objects | Staff Visitors Pupils | - Use of access equipment restricted to those trained and competent in use. - Maximum working height not exceeded- users waist should not rise above top step. - Staff to re-position steps rather than overstretch. - Floor surfaces suitable to use ( firm and even surface) - Persons using access equipment to wear appropriate footwear ( low heeled, non slip soles) - Securely stored when not in use to avoid unauthorised use. |  |  |
| Manual Handling Unstable / unwieldy loads Falls, falling objects | Staff Visitors Pupils | - Ensure shelving is not overloaded - Organise storage with heavy / bulky objects at lower levels. - Frequently used items stored at easily accessible locations. - Use team handling where necessary. - Staff to be trained in moving and handling techniques. |  |  |

| REVIEWS: |  |  |
| --- | --- | --- |
| DATE OF REVIEW: | REVIEWED BY: | COMMENTS: |
| DATE OF REVIEW: | REVIEWED BY: | COMMENTS: |
| DATE OF REVIEW: | REVIEWED BY: | COMMENTS: |

| ##### GENERIC RISK ASSESSMENT |  |  |  |  |
| --- | --- | --- | --- | --- |
| Establishment: | Assessment by: | Date: |  |  |
| Review Date: | Approved by: | Date: |  |  |
| WORK ACTIVITY (brief description) Opening and closing windows / adjusting blinds |  |  |  |  |

| ## Hazard / Risk | Who is at Risk? | Normal Control Measures (Brief description and/or reference to source of information). | Additional Control Measures (to take account of local/individual circumstances). | Risk Rating H/M/L |
| --- | --- | --- | --- | --- |
| Use of incorrect equipment (ledge, desk, chairs etc) Falls, falling objects | Staff Visitors Pupils | - Staff instructed not to use makeshift steps (chairs/ tables/ledges etc) - Provide remote means of opening high windows i.e. long handled poles or mechanical openers. |  |  |
| Defective equipment / Falls, falling objects | Staff Visitors Pupils | - All access equipment suitable for purpose, checked before use and maintained in good condition. - Reporting procedure in place for identified defective items with prompt removal /remedial action. |  |  |
| Falls from unprotected window | Staff Visitors Pupils | - Fit window opening limiters to all windows above ground level |  |  |
| Poorly maintained window falls from frame | Staff Visitors Pupils | - Ensure windows are maintained in a safe condition |  |  |

| REVIEWS: |  |  |
| --- | --- | --- |
| DATE OF REVIEW: | REVIEWED BY: | COMMENTS: |
| DATE OF REVIEW: | REVIEWED BY: | COMMENTS: |
| DATE OF REVIEW: | REVIEWED BY: | COMMENTS: |

| ##### GENERIC RISK ASSESSMENT |  |  |  |  |
| --- | --- | --- | --- | --- |
| Establishment: | Assessment by: | Date: |  |  |
| Review Date: | Approved by: | Date: |  |  |
| WORK ACTIVITY (brief description) Use of stepladder to replace defective fluorescent tube in a classroom. Task involves brief 2 hands working to remove the diffuser and tube. Light work of short duration. |  |  |  |  |

| ## Hazard / Risk | Who is at Risk? | Normal Control Measures (Brief description and/or reference to source of information). | Additional Control Measures (to take account of local/individual circumstances). | Risk Rating H/M/L |
| --- | --- | --- | --- | --- |
| Use of incorrect equipment (desk, chairs etc) Falls, falling objects | Staff Visitors Pupils | - Staff instructed not to use makeshift steps (chairs/ tables etc) - User to ensure access equipment is of adequate height. - Step ladder provided for users to access areas safely |  |  |
| Defective equipment, collapse of ladder falls, falling objects | Staff Visitors Pupils | - All access equipment suitable for purpose, checked before use and maintained in good condition. - Frequent documented checks on stepladder to ensure safe working condition ( 6 monthly / termly) - Reporting procedure in place for identified defective items with prompt removal /remedial action. |  |  |
| Inappropriate use of access equipment Overreaching, stretching etc. Uneven floor causing instability Falls, falling objects | Staff Visitors Pupils | - User trained in use of ladders for minor inspection / maintenance tasks. - Access equipment restricted to those trained and competent in use. - 2 person job, second person available at ground level to hand diffuser / tubes to. - Move furniture if needed to gain access. - Maximum working height not exceeded- users waist should not rise above top step or top of handrail. - Staff to re-position steps rather than overstretch. - Floor surfaces suitable to use ( firm and even surface) - Persons using access equipment to wear appropriate footwear ( low heeled, non slip soles) - Stepladder securely stored when not in use to avoid unauthorised use. |  |  |
| Persons colliding with ladder falls / falling objects | Staff Visitors Pupils | - Segregate work area - Provide warning signs, cones etc. if needed. |  |  |
| Manual Handling Unstable / unwieldy loads Falls, falling objects | Staff Visitors Pupils | - Use team handling where necessary. - Staff to be trained in moving and handling techniques. - Obtain help transporting ladder if needed |  |  |
| Contact with electricity electrocution, burns | Staff Visitors Pupils | - Switch off electrical supply or tape off and label light switch if this is not possible. |  |  |

| REVIEWS: |  |  |
| --- | --- | --- |
| DATE OF REVIEW: | REVIEWED BY: | COMMENTS: |
| DATE OF REVIEW: | REVIEWED BY: | COMMENTS: |
| DATE OF REVIEW: | REVIEWED BY: | COMMENTS: |

| ##### GENERIC RISK ASSESSMENT |  |  |  |  |
| --- | --- | --- | --- | --- |
| Establishment: | Assessment by: | Date: |  |  |
| Review Date: | Approved by: | Date: |  |  |
| WORK ACTIVITY (brief description) Use of ladder for light work of short duration (maximum 30 mins). |  |  |  |  |

| ## Hazard / Risk | Who is at Risk? | Normal Control Measures (Brief description and/or reference to source of information). | Additional Control Measures (to take account of local/individual circumstances). | Risk Rating H/M/L |
| --- | --- | --- | --- | --- |
| Use of incorrect equipment (desk, chairs etc) Falls, falling objects | Staff Visitors Pupils Contractors | - User to ensure access equipment is of adequate height. - Consideration given to hiring suitable equipment for specific jobs or where work is liable to take >30mins. |  |  |
| Defective equipment, collapse of ladder falls, falling objects | Staff Visitors Pupils Contractors | - All access equipment suitable for purpose, checked before use and maintained in good condition. - Frequent documented checks on ladder to ensure safe working condition ( 6 monthly / termly) - Reporting procedure in place for identified defective items with prompt removal /remedial action. |  |  |
| Inappropriate use of access equipment Overreaching, stretching etc. Uneven floor causing instability Falls, falling objects | Staff Visitors Pupils Contractors | - User trained in use of ladders for minor inspection / maintenance tasks. - Access equipment restricted to those trained and competent in use. - Maximum working height not exceeded. - Ensure correct angle of ladder ( 1M out for 4M up) - Secure ladder - No lone working when using ladders. - Staff to re-position ladder rather than overstretch. - Floor surfaces suitable to use ( firm and even surface) - Persons using access equipment to wear appropriate footwear ( low heeled, non slip soles) - Securely stored when not in use to avoid unauthorised use. |  |  |
| Persons colliding with ladder | Staff Visitors Pupils Contractors | - Segregate work area effectively. - Provide warning signs, cones or fencing if needed. - Schedule work to take place when persons are not in immediate area. |  |  |
| Manual Handling Unstable / unwieldy loads Falls, falling objects | Staff Visitors Pupils Contractors | - Accompanying tools and equipment carried in tool belt. Both hands to be kept free when climbing ladder. - Use team handling where necessary. - Staff to be trained in moving and handling techniques. - Obtain help transporting ladder if needed |  |  |
| Environmental conditions ( weather, wet ground, uneven surfaces etc.) | Staff Visitors Pupils Contractors | - Consideration given to environmental conditions prior to starting work. - No work to be undertaken in adverse weather conditions ( wind, rain, snow etc.) - Ensure rungs are clear of slippery substances, ice, mud etc. |  |  |

| REVIEWS: |  |  |
| --- | --- | --- |
| DATE OF REVIEW: | REVIEWED BY: | COMMENTS: |
| DATE OF REVIEW: | REVIEWED BY: | COMMENTS: |
| DATE OF REVIEW: | REVIEWED BY: | COMMENTS: |

| ##### GENERIC RISK ASSESSMENT |  |  |  |  |
| --- | --- | --- | --- | --- |
| Establishment: | Assessment by: | Date: |  |  |
| Review Date: | Approved by: | Date: |  |  |
| WORK ACTIVITY (brief description) Use of mobile tower scaffold . Use of equipment other than ladders justified – no protected platform, not possible to secure ladder. |  |  |  |  |

| ## Hazard / Risk | Who is at Risk? | Normal Control Measures (Brief description and/or reference to source of information). | Additional Control Measures (to take account of local/individual circumstances). | Risk Rating H/M/L |
| --- | --- | --- | --- | --- |
| Use of incorrect equipment Falls, falling objects | Staff Visitors Pupils Contractors | - User to ensure access equipment is of adequate height and suitable for task. - All users to be trained in work at height - Height of scaffold to be no more than 3 times base dimension ( or in accordance with manufacturers instructions) |  |  |
| Defective equipment, collapse of equipment, falls, falling objects | Staff Visitors Pupils Contractors | - All access equipment suitable for purpose, inspected before use and maintained in good condition. - Inspected weekly if it remains in place and after any event likely to have affected its stability. - Safe working load marked on equipment - To be erected only by certified persons. - Reporting procedure in place for identified defective items with prompt removal /remedial action |  |  |
| Inappropriate use of access equipment Overreaching, stretching etc. Uneven floor causing instability Falls, falling objects | Staff Visitors Pupils Contractors | - All users trained in use of mobile towers for inspection / maintenance tasks. - Access equipment restricted to those trained and competent in use. - Brakes to be applied in use. - Move tower from base only, all tools and persons removed from structure prior to moving. - Guardrails and toe boards to be provided. - No lone working when erecting or using tower. - Floor surfaces suitable to use ( firm and even surface) - Persons using access equipment to wear appropriate footwear ( low heeled, non slip soles) - Securely stored when not in use to avoid unauthorised use. Remove access ladder. |  |  |
| Persons or vehicles colliding with tower | Staff Visitors Pupils Contractors | - Segregate work area, create exclusion zone beneath work area - Provide warning signs, cones or temporary fencing if needed. - Schedule work to take place when persons are not in immediate area. |  |  |
| Contact with overhead electrical services Electrocution, burns | Staff Visitors Pupils Contractors | - Survey area prior to erection |  |  |
| Manual Handling Unstable / unwieldy loads Falls, falling objects | Staff Visitors Pupils Contractors | - Accompanying tools and equipment carried in tool belt. Both hands to be kept free when climbing ladder. - Use team handling where necessary. - Staff to be trained in moving and handling techniques. - Obtain help transporting equipment if needed |  |  |
| Environmental conditions ( weather, wet ground, uneven surfaces etc.) | Staff Visitors Pupils Contractors | - Consideration given to environmental conditions prior to starting work. - No work to be undertaken in adverse weather conditions ( wind, rain, snow etc.) - Ensure rungs are clear of slippery substances, ice, mud etc. |  |  |

| REVIEWS: |  |  |
| --- | --- | --- |
| DATE OF REVIEW: | REVIEWED BY: | COMMENTS: |
| DATE OF REVIEW: | REVIEWED BY: | COMMENTS: |
| DATE OF REVIEW: | REVIEWED BY: | COMMENTS: |

| ##### GENERIC RISK ASSESSMENT |  |  |  |  |
| --- | --- | --- | --- | --- |
| Establishment: | Assessment by: | Date: |  |  |
| Review Date: | Approved by: | Date: |  |  |
| WORK ACTIVITY (brief description) Work at height within drama |  |  |  |  |

| ## Hazard / Risk | Who is at Risk? | Normal Control Measures (Brief description and/or reference to source of information). | Additional Control Measures (to take account of local/individual circumstances). | Risk Rating H/M/L |
| --- | --- | --- | --- | --- |
| Defective equipment / Falls, falling objects | Staff Pupils Visitors | - All access equipment suitable for purpose, checked before use and maintained in good condition. - Inspected weekly if it remains in position. - To be erected only by certified persons. - Reporting procedure in place for identified defective items with prompt removal /remedial action. |  |  |
| Hit by falling equipment | Staff Pupils Visitors | - Segregate area under tower effectively, accompanying tools / equipment to be carried in tool belt. |  |  |
| Falls from mobile tower scaffold Inappropriate use, Overreaching, stretching etc. | Staff Pupils Visitors | - Pupils should not change stage lights - Consider installing lighting rigs which can be lowered to ground level - All relevant teaching staff trained in safe use of access equipment. Cascade to pupils, no pupils to use tower scaffold unless supervised by a competent person. - No lone working when erecting or using tower. - Persons using access equipment to wear appropriate footwear. - Code of conduct for pupil behaviour in drama studio. - Access equipment restricted to those competent in its safe use. Ensure access ladder removed / tower securely stored when not in use to prevent unauthorised use. - Drama dept has policy detailing safe working procedures for adjusting lights, working on gantry etc. |  |  |

| REVIEWS: |  |  |
| --- | --- | --- |
| DATE OF REVIEW: | REVIEWED BY: | COMMENTS: |
| DATE OF REVIEW: | REVIEWED BY: | COMMENTS: |
| DATE OF REVIEW: | REVIEWED BY: | COMMENTS: |

- LOW ROOF RISK ASSESSMENT

| Risk Assessment carried out by: |  |  |  |  | Date: |  |
| --- | --- | --- | --- | --- | --- | --- |
| Is there any history of students climbing on the roof during school hours? |  |  |  |  | Yes | No |
| Is there the potential for students to gain access to the roof during school hours? |  |  |  |  | Yes | No |
| HAZARDS | RISKS | RISK LEVEL | CONTROLS |  |  | RISK LEVEL |
| How or why do/could students get onto the roof? Consider the examples below and any others identified on your site. | What are the risks? For example: ## Fall trying to access the roof ## Fall from edge of roof - Fall through fragile roof materials - Fall through skylight - Fall from higher level (eg. second storey or chimney) which is accessible from the low roof | What is the level of risk before controls? High Medium Low | ## Can physical control be used to prevent climbing? For example: - Anti-climb fans around drainpipes - Apply anti-climb paint to area (above 2m with warning signs) - Remove fence, wall or railing - Remove trees and shrubs - Remove/secure moveable climbing aids in the area (wheelie bins etc) - Increase height of guard rails to external stairs - Ensure all fragile materials are identified and signed | Can management controls be implemented to prevent climbing? For example: ## School rules about not climbing/vandalising premises - Supervision at break times in area known climbing locations - No climbing signage in known areas - Ball games not allowed in areas close to low roofs - Procedures for safe retrieval of items from roof |  | What is the level of risk with controls? High Medium ### Low |
| Low roof compared to ground level |  |  |  |  |  |  |
| Drainpipe can be climbed |  |  |  |  |  |  |
| ## Window ledge provides foothold |  |  |  |  |  |  |
| Nearby fence, wall or railing |  |  |  |  |  |  |
| ## Tree or shrub close to building can be climbed |  |  |  |  |  |  |
| Low level porch or other structure |  |  |  |  |  |  |
| Moveable climbing aids available (eg. bins or outdoor furniture) |  |  |  |  |  |  |
| External fire escape stairs near to flat roof |  |  |  |  |  |  |
| “Allurement” on roof (eg. flagpole or chimney) |  |  |  |  |  |  |

| Are your staff required to access the roof as part of their duties? |  |  |  |  | YES | NO |
| --- | --- | --- | --- | --- | --- | --- |
| HAZARDS | RISKS | RISK LEVEL | CONTROLS |  |  | RISK LEVEL |
| Why do the staff need to access the roof? - Retrieve items - Maintenance - Access to plant or equipment How do staff get on the roof? - Ladder - Scaffold - Mobile Elevating work platform What do they do on the roof? - Work around the edge from a ladder or platform - Walk on the roof - Use tools and equipment | What are the risks? ## Fall trying to access the roof ## Fall from edge of roof - Fall through fragile roof materials - Fall through skylight | What is the level of risk before controls? High Medium Low | ## Can physical control be used to ensure safe access onto the roof? - Use long handled tools to help retrieve items from roof rather than actually climbing onto it - Ensure all fragile materials are identified and signed - Where there is a need for access across the roof to plant or equipment, a safe route should be identified and marked | Can management controls be implemented to ensure safe access to the roof? - Identify all reasons for staff needing to access the roof and carry out a risk assessment to ensure the most appropriate means of access is selected taking into account the frequency, duration and nature of the job - Reduce the need to retrieve items from the roof by not allowing ball games not allowed in areas close to low roofs - Develop safe procedures for activities on the roof (eg. retrieving items such as balls) |  | What is the level of risk with controls in place? High Medium Low |